CS350 Milestone 1

Width Modulation Lab

Matthew Biletnikoff

1. At what frequency can you see the LED start to blink?

The LED began to visibly blink when the PWM frequency was reduced to a low value where the switching became slow enough for the human eye to detect the on and off cycles. During testing, the LED appeared steady at higher frequencies such as sixty hertz. When the frequency was reduced to around ten hertz or lower, the blinking became clearly visible because the LED was turning on and off slowly enough for the eye to perceive each cycle.

2. At what duty cycle is the intensity of the LED perceptibly diminished from the initial fifty percent duty cycle?

The brightness of the LED noticeably decreased when the duty cycle dropped below the initial fifty percent level. Around forty percent the LED began to appear slightly dimmer, and the reduction in brightness became more obvious as the duty cycle continued to decrease. This happens because the LED receives power for a smaller portion of each PWM cycle.

3. When changing the duty cycle of the PWM, the loop used an increment of five every tenth of a second. Was this perceptibly smooth? If not, what could you change to improve the visual response? Why?

Using increments of five every tenth of a second produced a fairly smooth transition in brightness, but the change was still slightly stepped. A smoother visual response could be achieved by decreasing the increment size or reducing the delay between changes. For example, increasing the loop to adjust the duty cycle by increments of one instead of five would make the brightness change more gradually and appear smoother to the human eye.

4. What function sets the PWM frequency for a GPIO line?

The PWM frequency for a GPIO line is set using the function:

GPIO.PWM(pin, frequency)

In this lab the frequency was set using the command:

pwm18 = GPIO.PWM(18, 60)

which creates a PWM instance on GPIO pin eighteen with a frequency of sixty hertz

5. What function sets the duty cycle for a GPIO line?

The duty cycle for a GPIO line is controlled using the function:

pwm.ChangeDutyCycle(value)

This function adjusts the percentage of time that the signal remains on during each cycle. Increasing the duty cycle increases the brightness of the LED, while decreasing it makes the LED appear dimmer.